[image: image7.png]V7s)
{21505



          

	0.60mm Height 0603 Package

Hyper Red Chip LED

Technical Data Sheet


Part No.: YBH-S0603URC-T6-6A
Features:

Descriptions:

Applications:

Package Dimension:

[image: image1.wmf]Unit: mm

Tolerance: ¡À0.10mm

Polarity

Recommended Soldering Pad Dimensions

0.7

0.8

0.8

0.8

0.25 [0.010]

1.2 [0.047]

1 [0.039]

0.6 [0.024]

0.3 [0.012]

0.8 [0.032]

+

1. Soldering terminal may shift in x, y direction.

2. Polarity referring onto the cathode mark is reversed on the UR/HR/SR.

1.6 [0.063]

PCB

Resin

LED Die

Cathode Mark

Soldering

Terminal

1

2

1

2

1

2

0.3 [0.012]

0.3 [0.012]


	Part No.
	Chip Material
	Lens Color
	Source Color

	YBH-S0603URC-T6-6A
	AlGaInP
	Water Clear
	Hyper Red


Notes:
1. All dimensions are in millimeters (inches).

2. Tolerance is ± 0.10mm (.004″) unless otherwise specified.

3. Specifications are subject to change without notice.
Absolute Maximum Ratings at Ta=25℃
	Parameters
	Symbol
	Max.
	Unit

	Power Dissipation
	PD
	60
	mW

	Peak Forward Current

(1/10 Duty Cycle, 0.1ms Pulse Width)
	IFP
	100
	mA

	Continuous Forward Current
	IF
	25
	mA

	Reverse Voltage
	VR
	5
	V

	Electrostatic Discharge (HBM)
	ESD
	2000
	V

	Operating Temperature Range
	Topr
	-40℃ to +80℃

	Storage Temperature Range
	Tstg
	-40℃ to +85℃

	Soldering Temperature
	Tsld
	260℃ for 5 Seconds


Electrical Optical Characteristics at Ta=25℃
	Parameters
	Symbol
	Min.
	Typ.
	Max.
	Unit
	Test Condition

	Luminous Intensity *
	IV
	120
	145
	---
	mcd
	IF=20mA (Note 1)

	Viewing Angle *
	2θ1/2
	---
	130
	---
	Deg
	IF=20mA (Note 2)

	Peak Emission Wavelength
	λp
	---
	632
	---
	nm
	IF=20mA

	Dominant Wavelength
	λd
	---
	624
	---
	nm
	IF=20mA (Note 3)

	Spectral Line Half-Width
	△λ
	---
	20
	---
	nm
	IF=20mA

	Forward Voltage
	VF
	1.60
	2.00
	2.40
	V
	IF=20mA

	Reverse Current
	IR
	---
	---
	10
	µA
	VR=5V


Notes:

1. Luminous Intensity Measurement allowance is ± 10%.

2. θ1/2 is the off-axis angle at which the luminous intensity is half the axial luminous intensity.
3. The dominant wavelength (λd) is derived from the CIE chromaticity diagram and represents the single wavelength which defines the color of the device.
Typical Electrical / Optical Characteristics Curves

(25℃ Ambient Temperature Unless Otherwise Noted)

[image: image2.wmf]Spectrum Distribution

Relative Luminous Intensity (%)

Ta=25¡æ

0

400

100

Forward Current & Forward Voltage

Forward Current IF (mA)

Ta=25¡æ

0

10

20

30

40

50

1.4

1.6

1.8

2.0

2.2

2.4

Forward Voltage  VF (V)

300

500

600

700

800

Wavelength 

λ

p (nm)

75

50

25

Luminous Intensity &

Ambient Temperature

Relative Luminous Intensity (%)

1

1000

-60

Ambient Temperature Ta (¡æ)

100

10

-40

-20

0

20

40

60

80

100

Luminous Intensity & Forward Current

Relative Luminous Intensity (%)

1

1000

10

Forward Current IF (mA)

100

10

0

10

1

10

2

10

3

Forward Current Derating Curve

Forward Current IF (mA)

0

0

10

20

30

40

50

Ambient Temperature Ta (¡æ)

25

f=1KHz

Duty=1/10

Ta=25¡æ

20

40

60

80

100

2.6

Ta=25¡æ

Radiation Diagram

0.3

0

0.7

0.8

1.0

0.9

0.1

0

0.2

0.4

0.6

0.5

o

10

o

20

o

30

o

40

o

50

o

60

o

70

o

80

o

90

o


Reel Dimensions:

[image: image3.png]960.0£0.1
#178.0£0.1

2130205

- koos10
— 12.0¢1.0
Unit: mm

25mm




Carrier Tape Dimensions:
Loaded quantity 4000 PCS Per reel.
[image: image4.png]Progressive direction
Tgieeslve o e O

[ e

N
T b+ —d

L 7’ A

2.0+0.05 4.0  1.55+0.05

e el

+
Polarity





Please read the following notes before using the product:
1. Over-current-proof

Customer must apply resistors for protection, otherwise slight voltage shift will cause big current change (Burn out will happen).
2. Storage

2.1 Do not open moisture proof bag before the products are ready to use.

2.2 Before opening the package, the LEDs should be kept at 30℃ or less and 90%RH or less.

2.3 The LEDs should be used within a year.

2.4 After opening the package, the LEDs should be kept at 30℃ or less and 70%RH or less.

2.5 The LEDs should be used within 168 hours (7 days) after opening the package.

2.6 If the moisture adsorbent material (silica gel) has fabled away or the LEDs have exceeded the storage time, baking treatment should be performed using the following conditions. Baking treatment: 60±5℃ for 24 hours.
3. Soldering Condition

3.1 Pb-free solder temperature profile.
[image: image5.png]1~5°C/ sec.

TN

Pre-heating
1 ~5°C/ sec. 180 ~ 200°C

260°C Max.
5sec, Max.

120sec.Max|




3.2 Reflow soldering should not be done more than two times.

3.3 When soldering, do not put stress on the LEDs during heating.

3.4 After soldering, do not warp the circuit board.
4. Soldering Iron

Each terminal is to go to the tip of soldering iron temperature less than 260℃ for 5 seconds within once in less than the soldering iron capacity 25W. Leave two seconds and more intervals, and do soldering of each terminal. Be careful because the damage of the product is often started at the time of the hand solder.

5. Repairing

Repair should not be done after the LEDs have been soldered. When repairing is unavoidable, a double-head soldering iron should be used (as below figure). It should be confirmed beforehand whether the characteristics of the LEDs will or will not be damaged by repairing.

[image: image6.png]



6. Caution in ESD
Static Electricity and surge damages the LED. It is recommended to use a wrist band or anti-electrostatic glove when handling the LED. All devices, equipment and machinery must be properly grounded.
Package in 8mm tape on 7" diameter reel.


Compatible with automatic placement equipment.


Compatible with infrared and vapor phase reflow solder process.


Mono-color type.


The product itself will remain within RoHS compliant Version.





The S0603 SMD LED is much smaller than lead frame type components, thus enable smaller board size, higher packing density, reduced storage space and finally smaller equipment to be obtained.


Besides, lightweight makes them ideal for miniature applications, etc.





Automotive: Backlighting in dashboard and switch.


Telecommunication: Indicator and backlighting in telephone, sound and fax.


Flat backlight for LCD, switch and symbol.


General use.








Spec No.: S0603-T6          Rev No.: V.2          Date: Mar.2025-09-01        Page: 1 OF 8
Email:499200972@qq.com:  
Guangzhou Baihui Optoelectronic Technology Co., LTD           www.ledbaihui.com

_1280304403.dwg

_1278936176.dwg

